autokey.com.ua

INFINEON DAP INTERFACE

How to use

For configuration   --    CLOCK  = P10.9   and   DATA = P10.12

THE SAME PIN CONNECTION NAMES FOR 64-PIN CASE and FOR 64-PIN CASE.

STEP 1

Connect mcu pin  TRST  to mcu pin  +5V

STEP 2

Connect external GND and +12V module power  ,  power ON    and check  voltage on pins P10.0  P10.1  P10.2

voltage on configuration pins must be equal :

P10.0 = 0V

P10.1 = 0V

P10.2 = 5V

STEP 3

Disconnect external power and connect configurations pins  to  +5V  or  GND  pins if necessary

(PLEASE  LIFT configuration  pins BEFORE CONNACTION TO  +5V or GND  if your pins have wrong voltage or have direct connections to  GND or +5V  )

STEP 3

Connect programmer pins  to:

GND  INFINEON  adapter   to   mcu GND airbag

+5V INFINEON  adapter    to mcu +5V PIN

RESET INFINEON  adapter    to  mcu RESET

CLOCK INFINEON  adapter    to  P10.9

DATA INFINEON  adapter      to  P10.12

Connect external  12v module power  , run  INFINEON software and try to read

--------------------

For another configuration make connections according

INFINEON Application Note  AP16146

https://www.infineon.com/dgdl/AP1614611_XC2000_XE166family_pin_power_reset.pdf?folderId=db3a304313719f4f011372b30f58008a&fileId=db3a304323b87bc20123c37a7afa6b10&ack=t